FINAL TEST

Written exam for hydraulic course CETOP level II

Name_______________________________________

Company__________________________________

Date_____________________		Time available: 2 hours

Task 25 multiple choice questions

The student should start the final test on signal from the instructor

Mark the correct alternative(s) with an X

Wrong entry: Black out the wrong entry, and mark the correct alternative with an X.

Qualifying score: 60% (15 correct answers)

Date:							   Time available: 30 min.

Tasks:	20 Multiple Choice Questions

The student should start the final test on signal from the instructor.

Mark the correct alternative with an X.

Wrong entry: Black out the wrong entry, and mark the correct alternative with a new X.

Qualifying score: 70% (14 correct answers)

) correct answers

Date:							   Time available: 30 min.

Tasks:	20 Multiple Choice Questions

The student should start the final test on signal from the instructor.

Mark the correct alternative with an X.

Wrong entry: Black out the wrong entry, and mark the correct alternative with a new X.

Qualifying score: 70% (14 correct answers)

) correct answers

Formula sheet

Flow and velocity:

A = cm2
v = m/min

Q=A*v→a1*v1=A2*v2

Pressure:

p = bar
F = daN
A =cm2

p=FA

Flowrate:

V = L/min
A = cm2

c+=VA1*6

c-=VA2*6

Hydraulic power:

p = bar
V = L/min

P=p*V600

- The Flow Law

The flow rate Q1 entering a pipe, is equal to the flowrate Q2 leaving the pipe. The area A1 is 100 cm2, the velocity v1 is 51 m/min. and the area A2 60 cm2.

What is the velocity v2 of the fluid?

- 63 m/min

- 67 m/min

- 77 m/min

- 85 m/min

- Pressure

F = 500 daN

A1 = 15 cm2

A2 = 10 cm2

The flow in the system is 10 L/min. What are the 2 system pressures when the piston is moving in positive and negative direction?

- 30 bar positive / 45 bar negative

- 33 bar positive / 50 bar negative

- 35 bar positive / 52 bar negative

- 37 bar positive / 55 bar negative

- Flowrate

F = 500 daN

A1 = 15 cm2

A2 = 10 cm2

The flow in the system is 10 L/min. How fast will the piston move in positive and negative direction?

- 0,11 m/min positive / 0,16 m/min negative

- 1,1 m/min positive / 1,6 m/min negative

- 11 m/min positive / 16 m/min negative

- 7 m/min positive / 5,5 m/min negative

- Hydraulic power

The oil flow thru a motor is 100 L/min. and the system pressure is 240 bar.
What is the motors hydraulic power?

- 25 kW

- 37 kW

- 40 kW

- 42 kW

- Flow control

Which configuration would be most appropriate to use to control the piston speed in plus direction?

| No. 1 | No. 2 |
| --- | --- |
| No. 3 | No. 4 |

- No. 1

- No. 2

- No. 3

- No. 4

- Brake valve

Which pressure can be measured in point C?

- P = 200 bar

- P = 250 bar

- P = 300 bar

- P = 325 bar

- Brake valve

The valves opening pressure should be adjusted to a higher value than the pressure from the load. How much higher should the opening pressure be adjusted to?

- 1,10 x P

- 1,20 x P

- 1,25 x P

- 1,30 x P

- Hydraulic symbols

The 4 drawings show different hydraulic symbols. Write the drawing number next to the correct reference in the table below.

| No. 1 | No. 2 |
| --- | --- |
| No. 3 | No.4 |

| Valve no. | The correct term for the valve |
| --- | --- |
|  | Variable displacement pump with pressure compensator, 1 direction of flow, 1 direction of rotation, case drain port |
|  | Variable displacement pump, 2 directions of flow, 1 direction of rotation, case drain port |
|  | Fixed displacement pump/motor, 1 direction of flow, 1 direction of rotation |
|  | Fixed displacement motor, 2 directions of flow, 2 directions of rotation |

- Pumps

The 3 drawings show different pump compensating systems. Write the drawing number next to the correct reference in the table below.

| No. 1 |  | No. 2 |  |  |
| --- | --- | --- | --- | --- |
| No. 3 |  |  |  |  |
| Pump no. |  | The correct term for compensation |
|  |  | Pressure compensation |
|  |  | Flow compensation |
|  |  | Load sensing |

- Pump characteristic - Volumetric efficiency

A pump´s displacement is 50 L/min. But when testing the pump at 100 bar pressure, the flow is 41,5 L/min. What is the pumps volumetric efficiency at 100 bar?

- 0,72

- 0,75

- 0,81

- 0,83

- Pump Characteristic – Total efficiency.

The pump from previous task has a mechanical efficiency at 0,81. What is the pumps total efficiency at 100 bar?

- 0,65

- 0,67

- 0,81

- 0,82

- HI –LO pump

F = 2000 daN

When the piston is moving in positive direction, the friction, and weight of the piston generate a force of 100 daN. The system shall be supplied from both pumps. When the piston reaches the box, a force of 2000 daN is generated, and only 1 pump shall supply the system. 1 Gallon = 4,7 L

What would be the correct adjustment for pressure relieve valve 1 and 2?

| Pressure relive valve 1 |  | Pressure relieve valve 2 |  |
| --- | --- | --- | --- |
|  | >65 bar |  | >2 bar |
|  | >70 bar |  | >3 bar |
|  | >75 bar |  | >4 bar |
|  | >80 bar |  | >5 bar |

- Master and slave Cylinder

The flow into the master cylinder is 200 L/min.

The area of the piston on the master is 100 cm2. The piston rod is 25 cm2.

How fast will the slave cylinder move?

- 0,33 m/min.

- 1 m/min.

- 1,33 m/min.

- 1,5 m/min.

- The mussel diagram

The mussel diagram above shows a performance of a Danfoss ORBIT motor. The motor shall deliver a torque of 16 daNm. The oil flow is 40 L/min. How fast will the motor turn?

- 144 rpm

- 175 rpm

- 200 rpm

- 210 rpm

- The mussel diagram

Use the mussel diagram to answer the question.
If the oil flow is set to 30 L/min and the system pressure is 80 bar, how much power will the motor then deliver?

- 2 kW

- 3 kW

- 4 kW

- 5,1 kW

- Motor types

The 4 drawings show different motor types. Write the drawing number next to the correct reference in the table below.

| No. 1 | No. 2 |
| --- | --- |
| No. 3 | No.4 |

| Motor no. | The correct term for the motor |
| --- | --- |
|  | Gear motor |
|  | Vane motor |
|  | Swash plate motor |
|  | Radial piston motor |

- Hydraulic symbols

The 4 drawings show different hydraulic symbols. Write the drawing number next to the correct reference in the table below.

| No. 1 | No. 2 |
| --- | --- |
| No. 3 | No.4 |

| Valve no. | The correct term for the valve |
| --- | --- |
|  | Pressure relief valve, directly operated, internal pilot oil supply external drain port |
|  | Pressure relief valve, directly operated, internal pilot oil supply |
|  | 3-way pressure reducing valve, directly operated, internal pilot oil supply |
|  | 2-way pressure reducing valve, directly operated, internal pilot oil supply |

- Hydraulic symbols

The 4 drawings show different hydraulic symbols. Write the drawing number next to the correct reference in the table below.

| No. 1 | No. 2 |
| --- | --- |
| No. 3 | No.4 |

| Valve no. | The correct term for the valve |
| --- | --- |
|  | 2-way flow control valve, pressure compensated |
|  | Throttle check valve |
|  | 3-way flow control valve, pressure and temperature compensated |
|  | Flow divider |

- ISO 4406 cleanliness code

| Sample no. | Particles ≤ 4 µm / 100 ml | Particles ≤ 6 µm / 100 ml | Particles ≤ 14 µm / 100 ml | ISO Code |
| --- | --- | --- | --- | --- |
| 1 | 190.000 | 58.600 | 1.525 |  |
| 2 | 250.000 | 64.000 | 3.700 |  |
| 3 | 77.600 | 17.000 | 1.500 |  |

In the table above, is the result of 3 oil samples showing the number of found particles in different size.

Regarding to ISO 4406 insert the correct ISO Code for each oil sample

- Air in fluid

1

2

3

The 3 pictures show different kind of air in operating fluid. Write the drawing number next to the correct reference in the table below.

| Example no. | Form of air in the fluid |
| --- | --- |
|  | Air-in-oil dispersion |
|  | Surface foam |
|  | Dissolved air |

- Reservoir size

The size of a hydraulic tank for an open system should be:

- Same size as the pump flow

- 20 % bigger than the pump flow

- 3 – 4 times bigger than the pump flow

- 50 % bigger than the pump flow

- Accumulator types

The drawing shows different accumulator types. Write the drawing number next to the correct reference in the table below.

| Accumulator no. | The correct term for the accumulator |
| --- | --- |
|  | Bladder accumulator |
|  | Piston accumulator |
|  | Diaphragm accumulator |

- Working range for accumulator

The piston accumulator is commonly used for

- All round systems

- Low pressure systems

- Medium pressure systems

- High pressure systems

- Filter performance

Beta value

When testing the filter shown below, 100.000 particles are flowing upstream into the filter and only 1.000 particles are counted downstream.

What is the filters Beta value?

- 20

- 75

- 100

- 200

- Hydraulic cylinders

The 4 drawings show different hydraulic cylinders. Write the drawing number next to the correct reference in the table below.

| No. 1 | No. 2 |
| --- | --- |
| No. 3 | No.4 |

| Valve no. | The correct term for the valve |
| --- | --- |
|  | Telescopic hydraulic cylinder, double acting |
|  | Double acting single rod hydraulic cylinder, adjustable cushioning at both ends of piston |
|  | Single acting hydraulic cylinder, return stroke via pressurisation, piston chamber connected to tank |
|  | Telescopic hydraulic cylinder, single acting |